JEE MAIN REVISION MASTER: SEMICONDUCTOR DEVICES AND LOGIC GATES 2026 3 QUESTIONS 5% WEIGHTAGE ↑50.15%

Subject: Physics  |  Target: JEE Main  |  PYQs Analyzed: 32

# 1. High-Yield Concept Priority Matrix

| Priority Tier | Concept Name | Frequency | Category |
| --- | --- | --- | --- |
| Very High | p-n Junction Diode & Zener Diode Regulation | 19 PYQs | Semiconductor Diodes |
| High | Logic Gates & Truth Tables | 13 PYQs | Digital Electronics |

# 2. Master Formula Reference Card

| Concept / Topic | Mathematical Formulation & Standard Equations |
| --- | --- |
| p-n Junction Diode & Zener Diode Regulation | \text{Efficiency}_{\text{half-wave}} = 40.6\%, \quad \text{Efficiency}_{\text{full-wave}} = 81.2\%, \quad V_L = V_Z = \text{const}, \quad I_S = I_Z + I_L |
| Logic Gates & Truth Tables | \text{NAND: } Y = \overline{A \cdot B}, \quad \text{NOR: } Y = \overline{A + B}, \quad \overline{A \cdot B} = \overline{A} + \overline{B}, \quad \overline{A + B} = \overline{A} \cdot \overline{B} |

# 3. Concept Deep-Dive & Source PYQ Evidence

## 3.1 p-n Junction Diode & Zener Diode Regulation (Very High Yield — 19 PYQs)

Core Principle: Forward and reverse characteristics of p-n junctions, rectifiers, ripple frequency, and Zener diode shunt voltage stabilization.

⚠️ Common Trap / JEE Pitfall: In full-wave rectifiers, output ripple frequency is TWICE the AC input frequency (2f_(in)), whereas for half-wave it is f_(in).

💡 Speed Hack / Trick: In reverse breakdown, Zener diode maintains constant voltage V_Z across its terminals as long as I_Z ≥ I_(Z,min).

Source PYQs testing this concept:

[2026 Shift] Q#1 (MCQ): "A wafer of pure germanium crystal has two parts X and Y. The end X is obtained by doping with arsenic and Y with indium. It is connected to ..."  [Ans: N/A]

[2026 Shift] Q#2 (MCQ): "In which of the following figures, diode is reverse biased?..."  [Ans: N/A]

[2025 Shift] Q#1 (MCQ): "The circuit shown in figure contains two ideal diodes D_(1) and D_(2) . If a cell of emf 3V and negligible internal resistance is conn..."  [Ans: N/A]

## 3.2 Logic Gates & Truth Tables (High Yield — 13 PYQs)

Core Principle: Universal logic gates (NAND and NOR), De Morgan's theorems, and Boolean circuit reduction.

⚠️ Common Trap / JEE Pitfall: NAND and NOR gates are UNIVERSAL gates: any Boolean function can be constructed using only NAND or only NOR gates.

💡 Speed Hack / Trick: Connect inputs of a NAND or NOR gate together to obtain a NOT gate: \overlineA · A = \overlineA.

Source PYQs testing this concept:

[2026 Shift] Q#3 (MCQ): "An n-type and p-type semiconductor can be obtained by respectively doping pure silicon with..."  [Ans: N/A]

[2024 Shift] Q#3 (MCQ): "The upper level of valence band and lower level of conduction band overlap in the case of..."  [Ans: N/A]

[2023 Shift] Q#4 (MCQ): "A truth table for the given circuit is..."  [Ans: C]

# 4. Benchmark Illustrative PYQs & Master Solutions

Benchmark Problem 1: KCET 2026 (Q#1)

A wafer of pure germanium crystal has two parts X and Y. The end X is obtained by doping with arsenic and Y with indium. It is connected to a battery as shown in the figure. Which of the following statements is correct?

(A) X is p-type, Y is n-type and the junction is forward biased

(B) X is n-type, Y is p-type and the junction is forward biased

(C) X is p-type, Y is n-type and the junction is reverse biased

(D) X is n-type, Y is p-type and the junction is reverse biased

Correct Answer: None Solution Analysis: Part X is doped with arsenic. Arsenic is a Group 15 element, so it has 5 valence electrons. When it is added to germanium, it gives extra free electrons. So, part X becomes an n-type semiconductor. Part Y is doped with indium. Indium is a Group 13 element, so it has 3 valence electrons. When it is added to germanium, it creates holes. So, part Y becomes a p-type semiconductor. In the battery symbol, the longer line represents the positive terminal, and the shorter line represents the negative terminal. From the figure, part X is connected to the positive terminal and part Y is connected to the negative terminal. Since the n-type side is connected to the positive terminal and the p-type side is connected to the negative terminal, the p-n junction is reverse biased.

Benchmark Problem 2: KCET 2026 (Q#3)

An n-type and p-type semiconductor can be obtained by respectively doping pure silicon with

(A) Arsenic and Phosphorous respectively

(B) Indium and Aluminium respectively

(C) Phosphorous and Indium respectively

(D) Aluminium and Boron respectively

Correct Answer: None Solution Analysis: An n -type semiconductor is obtained by doping pure silicon with a pentavalent impurity, such as Phosphorus or Arsenic. A p -type semiconductor is obtained by doping pure silicon with a trivalent impurity, such as Boron, Aluminium, or Indium. So, for silicon: n-type → Phosphorus p-type → Indium Hence, the correct option is: Option C: Phosphorous and Indium respectively