JEE MAIN REVISION MASTER: SEMICONDUCTOR SYLLABUS REDUCED

Subject: Physics  |  Target: JEE Main  |  PYQs Analyzed: 92

# 1. High-Yield Concept Priority Matrix

| Priority Tier | Concept Name | Frequency | Category |
| --- | --- | --- | --- |
| Very High | Logic Gates & Truth Tables | 50 PYQs | Digital Electronics |
| Very High | p-n Junction Diode & Zener Diode Regulation | 44 PYQs | Semiconductor Diodes |

# 2. Master Formula Reference Card

| Concept / Topic | Mathematical Formulation & Standard Equations |
| --- | --- |
| Logic Gates & Truth Tables | \text{NAND: } Y = \overline{A \cdot B}, \quad \text{NOR: } Y = \overline{A + B}, \quad \overline{A \cdot B} = \overline{A} + \overline{B}, \quad \overline{A + B} = \overline{A} \cdot \overline{B} |
| p-n Junction Diode & Zener Diode Regulation | \text{Efficiency}_{\text{half-wave}} = 40.6\%, \quad \text{Efficiency}_{\text{full-wave}} = 81.2\%, \quad V_L = V_Z = \text{const}, \quad I_S = I_Z + I_L |

# 3. Concept Deep-Dive & Source PYQ Evidence

## 3.1 Logic Gates & Truth Tables (Very High Yield — 50 PYQs)

Core Principle: Universal logic gates (NAND and NOR), De Morgan's theorems, and Boolean circuit reduction.

⚠️ Common Trap / JEE Pitfall: NAND and NOR gates are UNIVERSAL gates: any Boolean function can be constructed using only NAND or only NOR gates.

💡 Speed Hack / Trick: Connect inputs of a NAND or NOR gate together to obtain a NOT gate: \overlineA · A = \overlineA.

Source PYQs testing this concept:

[2026 Evening Shift] Q#1 (MCQ): "The outputYfor the given inputsAandBto the circuit is :..."  [Ans: N/A]

[2026 Morning Shift] Q#1 (MCQ): "For the given logic circuit, which of the following inputs combination will make both LED-1 and LED-2 to glow?..."  [Ans: C]

[2026 Evening Shift] Q#1 (MCQ): "If X and Y are the inputs, the given circuit works as _________...."  [Ans: D]

## 3.2 p-n Junction Diode & Zener Diode Regulation (Very High Yield — 44 PYQs)

Core Principle: Forward and reverse characteristics of p-n junctions, rectifiers, ripple frequency, and Zener diode shunt voltage stabilization.

⚠️ Common Trap / JEE Pitfall: In full-wave rectifiers, output ripple frequency is TWICE the AC input frequency (2f_(in)), whereas for half-wave it is f_(in).

💡 Speed Hack / Trick: In reverse breakdown, Zener diode maintains constant voltage V_Z across its terminals as long as I_Z ≥ I_(Z,min).

Source PYQs testing this concept:

[2026 Morning Shift] Q#1 (MCQ): "Find the correct combination of A,B,C and D inputs which can cause the LED to glow...."  [Ans: D]

[2026 Evening Shift] Q#2 (MCQ): "Given below are two statements: one is labelled as Assertion A and the other is labelled as Reason R Assertion A : A diode under reverse-b..."  [Ans: C]

[2026 Morning Shift] Q#2 (MCQ): "For the given circuit (shown in part (A)) the time dependent input voltage v_(in)(t) and corresponding output v_(o)(t) are shown in part..."  [Ans: B]

# 4. Benchmark Illustrative PYQs & Master Solutions

Benchmark Problem 1: JEE Main 2026 (Online) 5th April Morning Shift (Q#1)

For the given logic circuit, which of the following inputs combination will make both LED-1 and LED-2 to glow?

(A) A=0,B=1,C=1

(B) A=1,B=0,C=0

(C) A=1,B=0,C=1

(D) A=1,B=1,C=0

Correct Answer: C Solution Analysis: Y_(1) is the output of an OR gate whose inputs are A and B , Y_(1)=A+B Y_(2) is an AND gate taking Y_(1) and C as input Y_(2)=(A+B) · C Y_(3) is an AND gate taking Y_(2) and A as input, Y_(3)=[(A+B) · C] · A Y_(3)=A · (A+B) · C=A · C So, the truth table is, A B C Y 1 = A + B Y 2 = (A + B) · C Y 3 = A · C 0 0 0 0 0 0 0 0 1 0 0 0 0 1 0 1 0 0 0 1 1 1 1 0 1 0 0 1 0 0 1 0 1 1 1 1 1 1 0 1 0 0 1 1 1 1 1 1 LED1 glows when Y_(1)=1 and LED2 glows when Y_(3)=1 So for both the LEDs to glow we need Y_(1)=1 and Y_(3)=1 From the table, this happens when: Since Y_(3)=A · C , so A=1,C=1 And Y_(1) will automatically be 1 because A=1 Hence, the possible combinations are ( A=1,B=0,C=1 ) and ( A=1,B=1,C=1 ). Thus, the correct option is (C).

Benchmark Problem 2: JEE Main 2026 (Online) 22nd January Morning Shift (Q#1)

Find the correct combination of A,B,C and D inputs which can cause the LED to glow.

(A) 1000

(B) 0011

(C) 0100

(D) 1101

Correct Answer: D Solution Analysis: Gate 1 is a NOR gate with both inputs tied together to A. A NOR gate with shorted inputs acts as a NOT gate. Y_(1)=bar(A+A)=bar(A) Inputs Output, Y 1 = A+ A = A 1 1 0 0 0 1 Gate 2 is also a NOR gate with both inputs tied to B , which acts as a NOT gate. Y_(2)=bar(B+B)=bar(B) Inputs Output, Y 2 = B+ B = B 1 1 0 0 0 1 Gate 3 is a NAND gate receiving inputs Y_(1) and Y_(2) . Y_(3)=bar(Y_(1) · Y_2) A B Y 1 = A Y 2 = B Y 1 · Y 2 Y_(3)=bar(Y_(1) · Y_2) 1 1 0 0 0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 0 1 1 1 0 Gate 4 is a NOR gate receiving inputs C and D . Y_(4)=bar(C+D) C D C + D Y 4 = C + D 1 1 1 0 1 0 1 0 0 1 1 0 0 0 0 1 For the LED to emit light, it must be forward-biased. The anode is connected to output of gate 1 and gate 2, i.e., Y_(3) . The cathode is connected to the output of gate 4 , i.e., Y_(4) . To achieve forward bias in a digital logic circuit, the anode must be at a HIGH voltage (1) and the cathode must be at a LOW voltage (0). Therefore, the required conditions are: Y_(3)=1∗Y_(4)=0 Options A B Y 3 (Anode) C D Y 4 (Cathode) ONN/OFF (1) 1 0 1 0 0 1 OFF (2) 0 0 0 1 1 0 OFF (3) 0 1 1 0 0 1 OFF (4) 1 1 1 0 1 0 ONN In case of 1101 , the anode is HIGH and the cathode is LOW. The LED is forward-biased and ON. Therefore, the correct option is (D).